Observation

Site Meeting & Discussion

On the 8th July 2013, a meeting was held on site to discuss and observe alleged damages with the following in attendance:

- Mr Les Millist of Structural Reporting Pty Ltd.

- Mr Derek Jorgensen of LMI Group (On Behalf of the Insured).

- Mr Peter Avnell of Cunningham Lindsey.

- Mr George Bejjani of Crawford & Company (On Behalf of the Insurers of 21 Maple Street Lugarno).

Discussions and inspection were held about the cause of damages in particular the following areas of concern:

External

- Copping tiles from the dwarf brick wall about the northern boundary and pathway.

- Northern boundary wall and abutting return retaining wall.

- External cracking to brickwork of north east corner.

- Cracking within concrete stair case tread.

- Cracking and settlement within pool rockery and paving.

Internal

- Internal cracking to laundry walls and cornices.

- Cracking within dining and living room ceiling/cornices.

- Cracking within bathroom ceiling and cornice.

- Cracking in bedroom & study walls and cornices.

- Cracking within garage wall.

Our observations and comments on these areas of concern are as follows:

External

Copping tiles from the dwarf brick wall about the northern boundary and pathway.

- Tiles were evidently missing from the wall which appears attributable to construction traffic traversing about the footpath and subsequently causing the detachment of the tiles.

Northern boundary wall and abutting return retaining wall.

- Initial concerns by the Insured and SR were raised by possible undermining of the boundary wall during construction and excavation works as depicted in SR Report Photograph 15.

- With the above mentioned concerns, SR and the Insured indicated that due to undermining, alleged cracking within the abutting retaining wall has been caused by associated movement and attachment to the northern wall.

- Inspection of the northern wall was completed from the neighbouring side to which I did not view any evidence of substantial cracking, movement or rotation of the wall associated with alleged undermining or vibration.

- Two (2) trees are growing out from the base of the northern wall as well as within the wall face itself.

- This has been noted in the SR report clause 2.4 and acknowledged as an issue causing long term performance and stability of the wall.

- Investigation of the tree roots clearly indicated that the root system has traversed along the back of the abutting return wall both laterally and vertically impacting on the bond of the brickwork over a prolonged period of time.

- Cracking about the lateral mortar joint and vertically through the brickwork was observed within the returning retaining wall to which SR had installed screws and tell-tale slides to monitor movement.

- At the time of this inspection, no tell-tale plates had cracked indicated any further movement from point of installation.

- At the time of this inspection, the Owners of 21 Maple Street had constructed a 230mm thick brick retaining wall founded off the bedrock stratum and backfilled with granular fill to support the footing of the northern brick wall.

- In my opinion, the constructed brick wall is satisfactory in capacity to facilitate support of the fill about the northern wall footing which has been constructed off the bedrock stratum.

- Furthermore, the abutting return wall is not structurally connected to the northern walls and is independent in formation, therefore is unaffected by any movement and unsupported as a factor in any crack propagation.

- Given the above, cracking within the abutting retaining wall appears a resultant of long term tree root pressures and articulation unrelated to any construction undermining.

External cracking to brickwork of north east corner.

- Minor hairline cracking was observed emanating from brickwork and mortar consistent with general articulation.

- At the time of this inspection, installed tell-tale plates had not cracked indicated any further movement from point of installation.

Cracking within concrete stair case tread.

- Minor hairline crack within exposed aggregate finished stair tread consistent with general articulation and age of construction.

Cracking and settlement within pool rockery and paving.

- General movement and settlement of pavers and rockery considered unrelated to construction works and a result of age and consolidation.

Internal

Internal cracking to laundry walls and cornices.

- Noted as per SR observations.

Cracking within dining and living room ceiling/cornices.

- Noted as per SR observations.

Cracking within bathroom ceiling and cornice.

- Cracking noted and cause clearly a building defect through the incorrect use of tile grout to fill between cornice and wall tiles rather than flexible sealant.

Cracking in bedroom & study walls and cornices.

- Noted as per SR observations.

Cracking within garage wall.

- Noted as per SR observations.

Discussion

In concluding cause of damage, SR reports in line with the Insured’s allegations that the documented cracking and movement are “a result of heavy ground borne vibration caused by the hydraulic rock excavation equipment and that the dwarf wall capping is damaged by construction traffic”.

It is noted that an initial dilapidation report prior to the commencement of any works was not completed by the Insured or the Owners of 21 Maple Street.

Given no photographic or written evidence of the condition of the building prior to works commencing, it is purely speculative to suggest by general consideration that damage has occurred as a result of vibration through the construction activity alone.

In attempt to verify such conclusion of causation through scientific and engineering basis without actual vibration monitoring results, possible vibration damage has been assessed against conditions, parameters and circumstances of this site and the building using relevant vibration standards and guidelines.

Vibration Criteria for Structural Damage to Buildings

In terms of relevant vibration damage criteria, Australian Standard AS2187.2 which refers to British Standard 7385: Part 2-1993 “Evaluation and measurement for vibration in buildings Part 2” is a definitive standard against which the likelihood of building damage from ground vibration can be assessed.

The standard sets guide values for building vibration based on the lowest vibration levels above which damage has been credibly demonstrated.

These levels are judged to give a minimum risk of vibration-induced damage, where minimal risk for a named effect is usually taken as a 95% probability of no effect.

Generated vibration from machinery and plant is identified as Peak Particle Velocity (PPV) (mm/s).

PPV is dependent on the following factors which have been considered in our evaluation of potential damage and are assessed against the following factors:

- Source and type of the vibration (Energy Level) – In this Case Operation of Hydraulic Hammer and Traversing of Excavator.

- Distance from the source.

- Ground condition (rock, clay and sand) – In this Case Rock.

Figure 1 Three Contributing Factors in Ground-Borne Vibration Problems

In assessing possible vibration damage in the natural ground condition of bedrock which is prevalent in the area, the following parameters have been explored.

Source and Type of Vibration (Energy Level) Generated From Hydraulic Hammer Operation

Sources of vibration which are considered in the standard include blasting, demolition, piling, ground treatments (e.g. compaction), construction equipment, tunnelling, road and rail traffic and industrial machinery.

From accounts and provided photographic evidence, the excavator used in constructions works within 21 Maple Street Lugarno was no greater than a 2.0t capacity excavator with hydraulic hammer attachment – Representative Excavator in Figure 2.

Figure 2 Small Excavator (No Greater than 2t Capacity) with Hammer Attachment

Such rock breakers and rock drills can generate relatively high vibration levels at close distances.

Typical ground vibration levels for an excavator with hammer attachment at various distances operating in hard sandstone has been obtained from various noise and vibration case studies  in Table 1 which in similar circumstances endeavour to formulate vibration effects.

Table 1 Typical Rock Breaker PPV Vibration Levels (mm/s) versus Distance

|  | PPV Vibration Level (mm/s) at Given Distance |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 m | 10 m | 20 m | 30 m | 40 m | 50 m |
| Heavy Rock Breaker (eg 1500 kg) | 4.50 | 1.30 | 0.40 | 0.20 | 0.14 | 0.10 |
| Medium Rock Breaker (eg 600 kg) | 0.2 | 0.06 | 0.02 | 0.01 | - | - |

With respect to equating produced PPV and building damages, reference is made to Table 2 which is found within AS2187.2 – 2006.

The guide values transient vibration judged to result in a minimal risk of cosmetic damage to residential buildings and industrial buildings are presented numerically.

Table 2 - Transient Vibration Guide Values for Cosmetic Damage (BS 7385-2)

| Line | Type of building | Peak component particle velocity in frequency range of predominant pulse |  |
| --- | --- | --- | --- |
|  |  | 4 Hz to 15 Hz | 15 Hz and above |
| 1 | Reinforced or framed structures. Industrial and heavy commercial buildings | 50 mm/s at 4 Hz and above |  |
| 2 | Unreinforced or light framed structure. Residential or light commercial type buildings | 15 mm/s at 4 Hz increasing to 20 mm/s at 15 Hz | 20 mm/s at 15 Hz increasing to 50 mm/s at 40 Hz and above |

BS 7385 states that the guide values in Table 2 relate predominantly to transient vibration which does not give rise to resonant responses in structures, and to low-rise buildings.

On this basis, should vibrational damage have been produced using an excavator, a PPV of no less than 15mm/s would be required to have emanated from the hammering activity.

Distance from the Source

The distances from the source (excavator with hammer attachment) at the northern boundary wall have been detailed below.

C

B

A

Figure 3 - Schematic Plan of Distances from the Source

Legend:

Source of Vibration (2t Excavator with Hydraulic Hammer at Northern Boundary Wall).

A – 	Approximately 8m from Living Room Wall.

B – 	Approximately 2m from Abutting Retaining Wall, External North East Brickwork and Laundry Wall.

C – 	Approximately 12m from Bedrooms and Study.

From this detail, distance from the source is less than 5 metres which current PPV values in Table 1 do not model such circumstances.

To establish PPV value at a distance of 2 metres which covers damages allegedly sustained to the abutting retaining wall, external north east brickwork and laundry wall, data has been logged and subsequently graphed to formulate working equation for vibration at this given distance.

Figure 3 Logged Rock Breaker PPV Vibration Levels versus Distance

The derived graph of vibration is an exponential curve that generates an empirical formula of y=2.8522.X- 1.662.

When simulating the equation, a resultant PPV of 0.9mm/s is produced at a given distance of 2 metres from the source.

Given the above outcome and a recognised PPV value of 15mm/s to cause cosmetic damage to building fabric, the source would need to be no less than 0.4 metres.

In this case, the only damage susceptible to vibration would be located at the northern boundary wall; however, this was not supported in my undertaken visual inspection or observations.

To this end, the following vibrational outcomes can be established from reference to the guides and equations:

- A minimum distance of 0.4 metres from the source to building fabric would be necessary to propagate a damaging PPV of 15mm/s.

- At 2 metres from the source, a PPV of 0.9mm/s has been calculated which is just susceptible (Bassett Acoustics, 2007) to humans and does not pose any cosmetic damage to buildings.

- Greater distanced building fabrics contained in laundry, living room, dining room and bedrooms fall well outside of generated PPV values of 15mm/s which would emanate damage.

Assessment of Vibration Impact from Traversing Excavator about Northern Footpath

In discounting cause of vibration from operating hammer alone, possible vibration generation from the traversing excavator has also been reviewed.

The graph below indicates assumed PPV values for such circumstances.

In our case, the value of the PPV generated at a distance of 2 metres is 0.8mm/s.

Figure 4 Vibrations from Typical Construction Operations (New 1986)

As with hammer vibration, traversing vibration of 0.8mm/s does not pose cosmetic damage to the building.

Conclusion

My instructions were to inspect and report on the following:

Please attend and provide us with a report as to the cause of sustained damages to the building.

The following findings are based upon the evidence presented and these should be read in conjunction with the preceding comments in the body of this report:

Proximate Cause & Effect

Damage as a Result of Vibration

In rationalising cause of damage to the building, the Insured and appointed Engineer Structural Reporting Pty Ltd (‘SR’) considers the damage to the building as “a result of heavy ground borne vibration caused by the hydraulic rock excavation equipment and that the dwarf wall capping is damaged by construction traffic”.

The SR conclusion is based on speculative consideration only without the premise or use of scientific or engineering formulation.

Furthermore, damages noted within the SR report have been recorded at time of inspection and not referenced against a pre-construction dilapidation report which would clearly delineate the pre-existing conditions of the building.

Given the benefit of alleged vibration damage, we have undertaken a theoretical review and calculation using guidelines and standards relevant to construction vibration and produced the following outcomes:

- The generated source from both the use of hydraulic hammer and excavator traversing about the northern boundary would produce Peak Particle Velocity (PPV) of 0.9mm/s which is well below the AS2187.2 guidelines necessary to emanate any damage to a building.

- Possible vibration damage may have occurred at a distance of 0.4m from the excavator hammer to propagate a damaging PPV of 15mm/s which could have occurred at the northern boundary fence; however this was not supported in site observations.

- A pre-construction dilapidation report is not on hand to verify the pre-existence of the alleged damages which would clearly outline origin of damages post construction.

- SR installed tell-tale plates did not crack or indicate any movement which is consistent with the vibration calculations presented.

Given the above mentioned, there is no physical or formulated evidence to indicate or support the claim that any damage from the construction activity in 21 Maple Street has caused or initiated any damage to the Insured’s building.

On this basis, alleged damages appear to be pre-existing relative to general movement, articulation and age.

Damage to the Northern Retaining Wall Abutting Return Wall

As indicated in observations, no physical evidence was presented to suggest that the northern wall had moved, cracked or deflected as a result of vibration or undermining during construction activity.

At present, the 230mm thick masonry retaining wall constructed to support the footings of the northern boundary wall is considered structurally acceptable to facilitate such function.

With respect to the cracking within the abutting return wall, crack propagation is clearly a result of long term tree root pressures from the two (2) trees growing beneath and within the wall structure.

The claim by the Insured and SR that the cause of cracking has evolved from the connection of the abutting wall to the northern boundary wall and subsidence is dismissed as a plausible causation given the walls are not connected and are independent in construction.

External Cracking of Brickwork to North Eastern Corner

Minor cracking within brickwork and mortar is clearly a result of general long term articulation and is un-defining to the structure.

Internal Cracking

As indicated in our vibration calculations, damage sustained to the internal cornices and linings are not considered a result of excavation operations.

The cracking to the wall linings and cornices are considered a result of creep and general articulation between differing materials not unusual given age of construction.

Damage is cosmetic in nature and can be easily repaired through flexible sealants and patching prior to repainting.

Sequence of Events

- The Insured has concerns with construction activity within 21 Maple Street, particular, vibration at the rear of the property as a result of rock excavation appointed Structural Reporting Pty Ltd (‘SR’) to undertake a Dilapidation Report on the 1st May 2013.

- A pre-construction dilapidation report was not undertaken by the Insured or the Owners of 21 Maple Street prior to the works commencing.

- SR re-attended the property on the 15th May 2013 further to allegations that additional damage had propagated as a result of the construction activity to which SR recorded further areas of damage and installed crack monitoring devices such as engineering screws and tell-tale glass plates over cracks.

- Inspection of premises was undertaken by me on 8th July 2013 which I viewed the damage and observed tell-tale glass plates had not cracked indicated no movement post installation had occurred.

Refer to Works Cited